承認書

SPECIFICATION FOR APPROVAL

| 客戶(Customer)： |  |
| --- | --- |
| 客戶料號(Customer P/N)： |  |
| 本厂型號(Supplier P/N)： | LLX3020B8-02 |
| 產品名稱(Description)： | 喇叭 |
| 產品規格(Specification)： | 3020 4Ω 3W |

| 客戶承認欄 (Customer Authorization Signature) | 電子 承認欄 (Supplier Audit) |  |  |
| --- | --- | --- | --- |
|  | 審核 | 檢查 | 制作 |
|  | 日期： 2025 年 12 月 13日 版次：A |  |  |

| Model No. | LLX3020B8-02 |  | Date | 2025-12-13 |  | Page No. | 1 / 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 產品規格內容 (SPECIFICATION) 1.使用範圍 (Scope) 此份規格書是針對多媒體產品內，所使用的微型揚聲器單體所提出的報告. This specification covers our product of miniature speaker unit for multimedia product. 2.标准测试条件(Test condition) 一般规格试验状态（特殊要求除外）：常温5~35℃，常湿45%~85%，气压860~1060hPa. 判定上有疑问时，试验状态为：温度20±2℃，相对湿度60%~70%，气压860~1060 hPa. The test conditions unless otherwise specified should be normal temperature 5 to 35 ℃,Normal humidity 45 to 85% and normal atmospheric pressure 860 to 1060 hPa,However,if there arises a doubt in judgment,the test conditions shall beas follows: Temperature 20±2℃ Humidity 60%~70% Atmosphere 860~1060 hPa 3.電聲特性 (ELECTRO-ACOUSTIC CHARACTERISTICS) 3.1額定阻抗(Rated Impedance): 4Ω ± 20% (AT 1.6K Hz ,1.0 Vrms) 3.2額定輸入功率(Rated Input Power) 1.8Watt (At 2.0 Vrms white noise 96 Hours) 3.3最大輸入功率(Max. Input Power) 3.0Watt 3.4最低共振頻率(Resonant Frequency) 1300 Hz ± 20% 3.5輸出音壓(Output S.P.L) 90 ± 3dB (1.0 Watt , 0.1 Meter. Average at 1.5K,2.0K,2.5K,3.0K Hz) 3.6實效頻率範圍(Frequency Range) Fo ~ 20KHz (By Output S.P.L. –10dB) 3.7失真率(Distortion) Less than 5% (At 1.6KHz ,3.4 Vrms) 3.8工作溫度(Operating Temperature) -25℃ ~ +50℃ 3.9儲存溫度(Storage temperature) -25℃ ~ +55℃ |  |  |  |  |  |  |  |
| 制 定 |  | 审 核 | 核 准 |  | 文 件 编 号 |  |  |

產品規格內容 (SPECIFICATION)

| Model No. | LLX3020B4-03 | Date | 2025-12-13 | Page No. | 2 / 6 |
| --- | --- | --- | --- | --- | --- |
| 4.可靠性測試 (Reliability Test) 4.1高低溫差循環測試(Temperature Cycling Test) A; Low Temperature (-25℃±3℃). B; High Temperature (+55℃±3℃). C; Number of Cycle (3 Cycles). D; Duration of Exposure 1 Hours, at Each Temperature Extremes. E; Duration of Recovery 1 Hours. 4.2抗濕度試驗(Humidity Test) A; 相對濕度(Relative Humidity); 90-95% at + 40℃ ± 3℃. B; 測試時間(Durations); 96 Hours. C; 恢覆時間(Durations of recovery); 6 Hours. 4.3連續負載測試(Load Test) White Noise Without Weighted Filter 4 Volts.(RMS) 100 Hrs. 4.4跌落試驗(Drop Test) 75 cm Free Falling On Concrete Floor, 10 Times. 4.5異常音測試(Buzzes & Rattles) Sine wave between at 4.0Vrms / 50~5000Hz 4.6極性測試(Polarity test) Diaphragm Shall Move Forward When Apply a Positive DC. Current to the “+”or “Marked” Terminal. |

產品規格內容 (SPECIFICATION)

| Model No. | LLX3020B8-02 | Date | 2025-12-13 | Page No. | 3 / 6 |
| --- | --- | --- | --- | --- | --- |
| 5.測量方式 (Measurement Method) |  |  |  |  |  |

產品規格內容 (SPECIFICATION)

| Model No. | LLX3020B8-02 | Date | 2025-12-13 | Page No. | 4/6 |
| --- | --- | --- | --- | --- | --- |
| 6. 頻率響應曲線圖(Frequency Response Curve) |  |  |  |  |  |

產品規格內容 (SPECIFICATION)

| Model No. | LLX3020B8-02 | Date | 2025-12-13 | Page No. | 5 / 6 |
| --- | --- | --- | --- | --- | --- |
| 7. 喇叭外觀描述 （Unit:mm tolerance:±0.3） |  |  |  |  |  |

產品規格內容 (SPECIFICATION)

| Model No. | LLX3020B8-02 | Date | 2025-12-13 | Page No. | 6/6 |
| --- | --- | --- | --- | --- | --- |
| 8.包裝示意圖(Packing Standard): （Unit:cm tolerance:±3） |  |  |  |  |  |